SVY 323 — COMPUTER APPLICATION I

GROUP ASSIGNMENT REPORT

Design and Development of a Closed Traverse Computation Programme in VB.NET

Prepared by: Group ____________

Members: [Name 1, Name 2, Name 3, Name 4, Name 5]

Level: Year 3 / Year 2   |   Programme: Surveying and Geoinformatics

Submitted to: Course Lecturer, SVY 323

Date of Submission: ____________________

# Table of Contents

# 1. Introduction

Traversing is one of the most widely used methods in plane surveying for establishing horizontal control and determining the relative positions of a series of connected survey stations. A traverse consists of a sequence of straight lines (legs) joining consecutive stations, whose lengths and directions are measured in the field using appropriate instruments such as a total station or theodolite and steel tape/EDM.

A closed traverse begins and ends at the same station, or at two known control stations, allowing the survey to be checked mathematically for angular and linear errors before the coordinates of the intermediate stations are accepted as final. Because traverse computation involves a large number of repetitive trigonometric calculations, it is well suited to automation using a computer programme.

This assignment required each group to design and code a computer programme, using Visual Basic (VB 2019 or later), capable of performing a complete closed-traverse computation: from raw field observations of included angles and distances, through to the computation of forward and back bearings, latitudes and departures (DN, DE), the Bowditch (compass rule) adjustment, final station coordinates (Northings and Eastings), the linear accuracy of the traverse, and the enclosed area of the traverse polygon.

This report presents the algorithm, flowchart, full programme source code, sample input data, computed output, and a discussion of the results obtained by the group.

# 2. Literature Review

Traverse computation is a foundational topic in plane surveying, treated extensively in standard surveying texts such as Uren and Price's "Surveying for Engineers" and Bannister, Raymond and Baker's "Surveying". These texts describe the traverse as a control network of connected lines whose directions are derived from measured angles and a known starting bearing, and whose lengths are obtained from taped or electronically measured distances.

The bearing of each leg is conventionally computed from the previous leg's back bearing using the included (interior or deflection) angle observed at each station; this is commonly referred to as the back-bearing (or forward computation) method. Once bearings and distances are known, each leg can be resolved into its north-south component (latitude, DN) and east-west component (departure, DE) using standard trigonometric relations, DN = L·cos(bearing) and DE = L·sin(bearing).

In a perfectly closed traverse, the algebraic sum of the latitudes and the algebraic sum of the departures should each equal zero. In practice, small residual errors (misclosures) always occur owing to instrumental, observational and natural errors. Several adjustment methods exist to distribute this misclosure among the traverse legs, of which the Bowditch Rule (Compass Rule) is the most widely taught and applied; it distributes the correction to each leg in proportion to the length of that leg relative to the total perimeter of the traverse. Other methods, such as the Transit Rule, distribute corrections in proportion to the latitude/departure components themselves, but the Bowditch Rule remains standard practice for traverses observed with equipment of uniform precision.

The overall reliability of a traverse is expressed as its linear (relative) accuracy, computed as the ratio of the linear misclosure to the total perimeter length, expressed in the form 1:X. Regulatory and professional surveying bodies specify minimum acceptable accuracy ratios depending on the class of survey being undertaken (e.g. 1:5,000 for general engineering surveys, up to 1:20,000 or better for control surveys).

With the growth of personal computing, the manual trigonometric computations traditionally performed with log tables or calculators have largely been replaced by dedicated software and spreadsheets. Visual Basic (VB), including its modern VB.NET incarnation within Microsoft Visual Studio, remains a popular teaching language for this purpose because of its simple, readable syntax, strong support for structured/procedural programming, and ease of building both console and graphical (Windows Forms) interfaces — making it well suited for coding traverse computation programmes of the kind required in this assignment.

# 3. Methodology

## 3.1 Overview of Approach

The group adopted a structured (procedural) programming approach in VB.NET, implemented as a Windows Forms (GUI) application so that field data can be entered, edited and computed interactively — well suited to a live class demonstration. The programme: (a) accepts the field data through an editable input grid — station names, included angles and leg distances, together with the starting bearing and coordinates carried forward from the initial control station; (b) performs the angular misclosure check; (c) computes forward and back bearings leg-by-leg; (d) computes latitudes and departures; (e) computes the linear misclosure and accuracy; (f) applies the Bowditch adjustment; (g) computes the final Northings and Eastings of every station; and (h) computes the enclosed area using the coordinate (shoelace) method — displaying every result instantly in an on-screen results grid and summary panel when the "Compute Traverse" button is clicked.

## 3.2 Algorithm

The step-by-step algorithm implemented by the programme is as follows:

- Step 1: Input the number of stations (n ≥ 5), station names, included angle at each station, and the distance of each leg; also input the starting bearing (from the initial control station) and the starting coordinates (N, E).

- Step 2: Sum the included angles and compare with the theoretical sum (n − 2) × 180° for a closed polygon traverse; compute and report the angular misclosure.

- Step 3: Starting from the given bearing of the first leg, compute the back bearing (± 180°) and then the forward bearing of the next leg using: Forward Bearing = Back Bearing − Included Angle (± 360° as necessary). Repeat for every leg around the traverse.

- Step 4: For each leg, compute the Latitude, DN = Distance × cos(Forward Bearing), and the Departure, DE = Distance × sin(Forward Bearing).

- Step 5: Sum all DN values (ΣDN) and all DE values (ΣDE) to obtain the linear misclosures eN and eE. Compute the linear misclosure = √(eN² + eE²) and the linear accuracy ratio = Perimeter ÷ Linear Misclosure, expressed as 1:X.

- Step 6: Apply the Bowditch Rule: correction to DN (or DE) for a given leg = −(eN or eE) × (leg distance ÷ total perimeter). Add these corrections to the raw DN/DE to obtain the adjusted DN/DE for each leg.

- Step 7: Starting from the known coordinates of the initial control station, successively add the adjusted DN and DE of each leg to obtain the Final Northing and Final Easting of every station.

- Step 8: Compute the enclosed area of the traverse polygon using the coordinate (shoelace) method: Area = ½ |Σ (E(i)·N(i+1) − E(i+1)·N(i))|.

- Step 9: Output all computed quantities — angular misclosure, bearings, DN/DE, corrections, adjusted DN/DE, final coordinates, linear accuracy and area.

## 3.3 Flowchart

The flowchart below summarises the logical flow of the traverse computation programme, from data input through to the final output of adjusted coordinates, accuracy and area.

## 3.4 User Interface

The programme presents an editable input grid for the station data (name, included angle in D M S, and distance to the next station), text boxes for the starting bearing and starting coordinates, "Add Row" / "Remove Row" controls so the traverse can be extended beyond five legs, and a "Compute Traverse" button. On computation, a results grid displays the bearing, DN/DE, Bowditch correction and final coordinate for every leg, alongside a summary panel showing the angular misclosure, linear misclosure, accuracy ratio and enclosed area. A mock-up of the interface, populated with the sample dataset used in this report, is shown below.

Figure: Windows Forms user interface of the traverse computation programme (populated with the sample dataset).

## 3.5 Assumptions

- The traverse is a closed-loop (polygon) traverse of five stations (A, B, C, D, E), closing back onto the initial control station A, satisfying the minimum of five included angles and associated distances specified in the assignment.

- The bearing of the first leg (A–B) is assumed known, having been derived from a back-bearing observation to a nearby reference/control point.

- The coordinates of station A are assumed known (carried forward from the initial control station).

- Included angles are interior angles measured clockwise at each station.

- All angular observations are assumed free of gross (blunder) errors; only small residual (accidental) errors requiring Bowditch adjustment are present.

# 4. Programme Code (VB.NET, VB 2019 or later)

The complete Visual Basic source code implementing the algorithm above is provided below. It was developed and tested as a Windows Forms Application in Visual Studio 2019, with the entire user interface built programmatically in a single code file (Form1.vb) — no separate Designer file is required.

'==================================================================================

'  SVY 323 - COMPUTER APPLICATION I

'  TRAVERSE COMPUTATION PROGRAMME (Closed Traverse) - WINDOWS FORMS (GUI) VERSION

'  Language : Visual Basic .NET (VB 2019 or higher) - Windows Forms App

'

'  HOW TO USE THIS FILE

'  ---------------------

'  1. In Visual Studio: File > New > Project > "Windows Forms App" (Visual Basic).

'  2. In Solution Explorer, DELETE the auto-generated "Form1.Designer.vb" file

'     (right-click it > Delete) - it is not needed because this single file

'     builds the whole user interface in code.

'  3. Open the auto-generated "Form1.vb" and REPLACE its entire contents with

'     everything in this file.

'  4. Press F5 (Start) to run. The input grid is pre-loaded with sample data

'     for five stations (A-B-C-D-E) - click "Compute Traverse" to see results,

'     or edit the grid with your own group's field data first.

'

'  WHAT THE PROGRAMME DOES

'  ------------------------

'   1. Angular misclosure check       (Sum of interior angles vs (n-2)*180)

'   2. Forward & back bearings        for every traverse leg

'   3. Latitudes (DN) & Departures(DE) for every leg

'   4. Linear misclosure & accuracy   (1 : X)

'   5. Bowditch (Compass Rule) corrections, proportional to leg length

'   6. Adjusted DN/DE and Final Northings & Eastings of every station

'   7. Area of the closed traverse polygon (Coordinate / Shoelace method)

'==================================================================================

Imports System

Imports System.Windows.Forms

Imports System.Drawing

Imports System.Collections.Generic

Public Class Form1

Inherits Form

' ---------------- Controls ----------------

Private WithEvents btnCompute As Button

Private WithEvents btnLoadSample As Button

Private WithEvents btnAddRow As Button

Private WithEvents btnRemoveRow As Button

Private inputGrid As DataGridView

Private resultsGrid As DataGridView

Private txtStartBearing As TextBox

Private txtStartN As TextBox

Private txtStartE As TextBox

Private lblAngularSum As Label

Private lblAngularMisclosure As Label

Private lblLinearMisclosure As Label

Private lblAccuracy As Label

Private lblArea As Label

' ---------------- Result structure ----------------

Structure LegResult

Public FromStation As String

Public ToStation As String

Public IncludedAngle As Double

Public Distance As Double

Public ForwardBearing As Double

Public BackBearing As Double

Public DN As Double

Public DE As Double

Public CorrDN As Double

Public CorrDE As Double

Public AdjDN As Double

Public AdjDE As Double

Public FinalN As Double

Public FinalE As Double

End Structure

Public Sub New()

BuildUserInterface()

LoadSampleData()

End Sub

' ==================================================================================

'  UI CONSTRUCTION (done entirely in code - no Designer.vb file required)

' ==================================================================================

Private Sub BuildUserInterface()

Me.Text = "SVY 323 - Closed Traverse Computation Programme"

Me.Width = 1150

Me.Height = 800

Me.StartPosition = FormStartPosition.CenterScreen

Me.Font = New Font("Segoe UI", 9)

' ---- Top panel: start bearing / start coordinates ----

Dim lblHeader As New Label With {

.Text = "Starting Control Data (carried forward from initial control station):",

.Left = 15, .Top = 12, .Width = 500, .Font = New Font("Segoe UI", 9, FontStyle.Bold)

}

Me.Controls.Add(lblHeader)

Dim lbl1 As New Label With {.Text = "Start Bearing (D M S):", .Left = 15, .Top = 40, .Width = 150}

txtStartBearing = New TextBox With {.Left = 170, .Top = 37, .Width = 100, .Text = "60 0 0"}

Dim lbl2 As New Label With {.Text = "Start Northing (m):", .Left = 290, .Top = 40, .Width = 130}

txtStartN = New TextBox With {.Left = 420, .Top = 37, .Width = 90, .Text = "1000.000"}

Dim lbl3 As New Label With {.Text = "Start Easting (m):", .Left = 525, .Top = 40, .Width = 120}

txtStartE = New TextBox With {.Left = 645, .Top = 37, .Width = 90, .Text = "1000.000"}

Me.Controls.AddRange({lbl1, txtStartBearing, lbl2, txtStartN, lbl3, txtStartE})

' ---- Buttons ----

btnLoadSample = New Button With {.Text = "Load Sample Data", .Left = 760, .Top = 35, .Width = 150, .Height = 28}

btnAddRow = New Button With {.Text = "Add Row", .Left = 920, .Top = 35, .Width = 90, .Height = 28}

btnRemoveRow = New Button With {.Text = "Remove Row", .Left = 1015, .Top = 35, .Width = 105, .Height = 28}

Me.Controls.AddRange({btnLoadSample, btnAddRow, btnRemoveRow})

' ---- Input grid ----

Dim lblInput As New Label With {

.Text = "Field Data Input (Station, Included Angle in D M S, Distance to NEXT station in metres) - minimum 5 rows:",

.Left = 15, .Top = 75, .Width = 750, .Font = New Font("Segoe UI", 9, FontStyle.Bold)

}

Me.Controls.Add(lblInput)

inputGrid = New DataGridView With {

.Left = 15, .Top = 100, .Width = 1105, .Height = 150,

.Anchor = AnchorStyles.Top Or AnchorStyles.Left Or AnchorStyles.Right,

.AllowUserToAddRows = False,

.RowHeadersVisible = False

}

inputGrid.Columns.Add("Station", "Station")

inputGrid.Columns.Add("Angle", "Included Angle (D M S)")

inputGrid.Columns.Add("Distance", "Distance to Next Stn (m)")

inputGrid.Columns(0).Width = 340

inputGrid.Columns(1).Width = 340

inputGrid.Columns(2).Width = 340

Me.Controls.Add(inputGrid)

' ---- Compute button ----

btnCompute = New Button With {

.Text = "COMPUTE TRAVERSE", .Left = 15, .Top = 260, .Width = 220, .Height = 36,

.Font = New Font("Segoe UI", 9, FontStyle.Bold), .BackColor = Color.LightSteelBlue

}

Me.Controls.Add(btnCompute)

' ---- Summary labels ----

lblAngularSum = New Label With {.Left = 260, .Top = 265, .Width = 420, .Text = "Sum of Angles / Misclosure: -"}

lblAngularMisclosure = New Label With {.Left = 260, .Top = 285, .Width = 420, .Text = ""}

lblLinearMisclosure = New Label With {.Left = 700, .Top = 265, .Width = 420, .Text = "Linear Misclosure: -"}

lblAccuracy = New Label With {.Left = 700, .Top = 285, .Width = 420, .Text = "Accuracy: -"}

lblArea = New Label With {.Left = 700, .Top = 305, .Width = 420, .Text = "Area: -", .Font = New Font("Segoe UI", 9, FontStyle.Bold)}

lblAngularSum.Font = New Font("Segoe UI", 9, FontStyle.Bold)

lblLinearMisclosure.Font = New Font("Segoe UI", 9, FontStyle.Bold)

lblAccuracy.Font = New Font("Segoe UI", 9, FontStyle.Bold)

Me.Controls.AddRange({lblAngularSum, lblAngularMisclosure, lblLinearMisclosure, lblAccuracy, lblArea})

' ---- Results grid ----

Dim lblResults As New Label With {

.Text = "Results: Bearings, Latitudes/Departures, Bowditch Corrections and Final Coordinates",

.Left = 15, .Top = 340, .Width = 750, .Font = New Font("Segoe UI", 9, FontStyle.Bold)

}

Me.Controls.Add(lblResults)

resultsGrid = New DataGridView With {

.Left = 15, .Top = 365, .Width = 1105, .Height = 380,

.Anchor = AnchorStyles.Top Or AnchorStyles.Bottom Or AnchorStyles.Left Or AnchorStyles.Right,

.AllowUserToAddRows = False,

.ReadOnly = True,

.RowHeadersVisible = False

}

Dim cols() As String = {"Leg", "Angle", "Dist(m)", "Fwd Bearing", "Back Bearing",

"DN(m)", "DE(m)", "Corr.DN", "Corr.DE", "Adj.DN", "Adj.DE",

"To Stn", "Final N", "Final E"}

For Each c As String In cols

resultsGrid.Columns.Add(c, c)

Next

Me.Controls.Add(resultsGrid)

End Sub

' ==================================================================================

'  SAMPLE DATA

' ==================================================================================

Private Sub LoadSampleData() Handles btnLoadSample.Click

inputGrid.Rows.Clear()

inputGrid.Rows.Add("A", "108 0 0", "100.02")

inputGrid.Rows.Add("B", "108 0 0", "99.97")

inputGrid.Rows.Add("C", "108 0 0", "100.05")

inputGrid.Rows.Add("D", "108 0 0", "99.94")

inputGrid.Rows.Add("E", "108 0 0", "100.03")

txtStartBearing.Text = "60 0 0"

txtStartN.Text = "1000.000"

txtStartE.Text = "1000.000"

End Sub

Private Sub AddRow_Click() Handles btnAddRow.Click

inputGrid.Rows.Add("", "0 0 0", "0.00")

End Sub

Private Sub RemoveRow_Click() Handles btnRemoveRow.Click

If inputGrid.Rows.Count > 0 And inputGrid.CurrentRow IsNot Nothing Then

inputGrid.Rows.Remove(inputGrid.CurrentRow)

End If

End Sub

' ==================================================================================

'  MAIN COMPUTATION - fired when "COMPUTE TRAVERSE" is clicked

' ==================================================================================

Private Sub ComputeTraverse_Click() Handles btnCompute.Click

Try

Dim rowCount As Integer = inputGrid.Rows.Count

If rowCount < 5 Then

MessageBox.Show("Please enter at least 5 stations (5 included angles and 5 leg distances) as required by the assignment.",

"Insufficient Data", MessageBoxButtons.OK, MessageBoxIcon.Warning)

Return

End If

Dim n As Integer = rowCount

Dim stationNames(n - 1) As String

Dim includedAngles(n - 1) As Double

Dim distances(n - 1) As Double

For i As Integer = 0 To n - 1

stationNames(i) = CStr(inputGrid.Rows(i).Cells(0).Value).Trim()

includedAngles(i) = ParseDMS(CStr(inputGrid.Rows(i).Cells(1).Value))

distances(i) = Convert.ToDouble(inputGrid.Rows(i).Cells(2).Value)

Next

Dim startBearing As Double = ParseDMS(txtStartBearing.Text)

Dim startN As Double = Convert.ToDouble(txtStartN.Text)

Dim startE As Double = Convert.ToDouble(txtStartE.Text)

' ---------------- 1. Angular misclosure ----------------

Dim sumAngles As Double = 0

For Each a As Double In includedAngles

sumAngles += a

Next

Dim theoreticalSum As Double = (n - 2) * 180.0

Dim angularMisclosure As Double = sumAngles - theoreticalSum

lblAngularSum.Text = "Sum of Angles = " & DecimalToDMS(sumAngles) & "   (Theoretical = " & DecimalToDMS(theoreticalSum) & ")"

lblAngularMisclosure.Text = "Angular Misclosure = " & DecimalToDMS(angularMisclosure)

' ---------------- 2. Bearings, DN, DE ----------------

Dim legs(n - 1) As LegResult

Dim fb As Double = startBearing

For i As Integer = 0 To n - 1

Dim fromStn As String = stationNames(i)

Dim toStn As String = stationNames((i + 1) Mod n)

If i > 0 Then

Dim bbPrev As Double = (legs(i - 1).ForwardBearing + 180.0) Mod 360.0

fb = (bbPrev - includedAngles(i)) Mod 360.0

If fb < 0 Then fb += 360.0

End If

Dim bb As Double = (fb + 180.0) Mod 360.0

Dim dist As Double = distances(i)

Dim rad As Double = fb * Math.PI / 180.0

legs(i).FromStation = fromStn

legs(i).ToStation = toStn

legs(i).IncludedAngle = includedAngles(i)

legs(i).Distance = dist

legs(i).ForwardBearing = fb

legs(i).BackBearing = bb

legs(i).DN = dist * Math.Cos(rad)

legs(i).DE = dist * Math.Sin(rad)

Next

' ---------------- 3. Linear misclosure & accuracy ----------------

Dim sumDN As Double = 0, sumDE As Double = 0, perimeter As Double = 0

For i As Integer = 0 To n - 1

sumDN += legs(i).DN

sumDE += legs(i).DE

perimeter += legs(i).Distance

Next

Dim linearMisclosure As Double = Math.Sqrt(sumDN ^ 2 + sumDE ^ 2)

Dim accuracyDenominator As Double = If(linearMisclosure = 0, 0, perimeter / linearMisclosure)

lblLinearMisclosure.Text = String.Format("Linear Misclosure = {0:0.0000} m  (eN={1:0.0000}, eE={2:0.0000})", linearMisclosure, sumDN, sumDE)

lblAccuracy.Text = If(linearMisclosure = 0, "Accuracy = Perfect Closure", String.Format("Linear Accuracy = 1 : {0:0}", accuracyDenominator))

' ---------------- 4. Bowditch adjustment ----------------

For i As Integer = 0 To n - 1

legs(i).CorrDN = -sumDN * (legs(i).Distance / perimeter)

legs(i).CorrDE = -sumDE * (legs(i).Distance / perimeter)

legs(i).AdjDN = legs(i).DN + legs(i).CorrDN

legs(i).AdjDE = legs(i).DE + legs(i).CorrDE

Next

' ---------------- 5. Final coordinates ----------------

Dim finalN As New Dictionary(Of String, Double)

Dim finalE As New Dictionary(Of String, Double)

finalN(stationNames(0)) = startN

finalE(stationNames(0)) = startE

Dim curN As Double = startN

Dim curE As Double = startE

For i As Integer = 0 To n - 1

curN += legs(i).AdjDN

curE += legs(i).AdjDE

legs(i).FinalN = curN

legs(i).FinalE = curE

finalN(legs(i).ToStation) = curN

finalE(legs(i).ToStation) = curE

Next

' ---------------- 6. Area (Shoelace / coordinate method) ----------------

Dim area As Double = 0

For i As Integer = 0 To n - 1

Dim s1 As String = stationNames(i)

Dim s2 As String = stationNames((i + 1) Mod n)

area += finalE(s1) * finalN(s2) - finalE(s2) * finalN(s1)

Next

area = Math.Abs(area) / 2.0

lblArea.Text = String.Format("Area = {0:0.000} sq.m  ({1:0.0000} hectares)", area, area / 10000.0)

' ---------------- 7. Populate results grid ----------------

resultsGrid.Rows.Clear()

For i As Integer = 0 To n - 1

resultsGrid.Rows.Add(

legs(i).FromStation & "-" & legs(i).ToStation,

DecimalToDMS(legs(i).IncludedAngle),

Format(legs(i).Distance, "0.00"),

DecimalToDMS(legs(i).ForwardBearing),

DecimalToDMS(legs(i).BackBearing),

Format(legs(i).DN, "0.0000"),

Format(legs(i).DE, "0.0000"),

Format(legs(i).CorrDN, "0.0000"),

Format(legs(i).CorrDE, "0.0000"),

Format(legs(i).AdjDN, "0.0000"),

Format(legs(i).AdjDE, "0.0000"),

legs(i).ToStation,

Format(legs(i).FinalN, "0.000"),

Format(legs(i).FinalE, "0.000")

)

Next

Catch ex As Exception

MessageBox.Show("Error in input data: " & ex.Message, "Computation Error", MessageBoxButtons.OK, MessageBoxIcon.Error)

End Try

End Sub

' ==================================================================================

'  HELPER FUNCTIONS

' ==================================================================================

''' <summary>Parses a "D M S" (degrees minutes seconds, space separated) or a

''' plain decimal-degree string into decimal degrees.</summary>

Private Function ParseDMS(text As String) As Double

text = text.Trim()

If text.Contains(" ") Then

Dim parts() As String = text.Split({" "c}, StringSplitOptions.RemoveEmptyEntries)

Dim deg As Double = Convert.ToDouble(parts(0))

Dim mins As Double = If(parts.Length > 1, Convert.ToDouble(parts(1)), 0)

Dim secs As Double = If(parts.Length > 2, Convert.ToDouble(parts(2)), 0)

Dim sign As Double = If(deg < 0, -1.0, 1.0)

Return sign * (Math.Abs(deg) + mins / 60.0 + secs / 3600.0)

Else

Return Convert.ToDouble(text)

End If

End Function

''' <summary>Formats decimal degrees as D°MM'SS.S" for display.</summary>

Private Function DecimalToDMS(decimalDeg As Double) As String

Dim d As Double = decimalDeg

Dim deg As Integer = CInt(Math.Truncate(d))

Dim minFull As Double = Math.Abs(d - deg) * 60.0

Dim mins As Integer = CInt(Math.Truncate(minFull))

Dim secs As Double = (minFull - mins) * 60.0

Return String.Format("{0}{1}'{2:00.0}""", deg.ToString() & Chr(176), mins.ToString("00"), secs)

End Function

End Class

' ==================================================================================

'  APPLICATION ENTRY POINT

'  (Visual Studio's Windows Forms App template normally provides this

'   automatically in "Program.vb" / "ApplicationEvents.vb". If your project

'   does not already start Form1 automatically, make sure "Form1" is set as

'   the Startup Form under Project Properties > Application.)

' ==================================================================================

# 5. Input Data

The sample field data used to test and demonstrate the programme is summarised in the table below. The starting bearing of line A–B is 60°00'00", and the coordinates of the initial control station A are Northing = 1000.000 m, Easting = 1000.000 m.

| Station | Included Angle | Leg | Distance (m) |
| --- | --- | --- | --- |
| A | 108°00'00" | A-B | 100.02 |
| B | 108°00'00" | B-C | 99.97 |
| C | 108°00'00" | C-D | 100.05 |
| D | 108°00'00" | D-E | 99.94 |
| E | 108°00'00" | E-A | 100.03 |

# 6. Results / Output Data

## 6.1 Angular Misclosure

Sum of included angles = 540°00'00"; theoretical sum (n−2)×180° = 540°00'00"; angular misclosure = 0°00'00" (well within acceptable tolerance).

## 6.2 Forward/Back Bearings, Latitudes (DN) and Departures (DE)

| Leg | Angle | Distance(m) | Fwd Bearing | Back Bearing | DN (m) | DE (m) |
| --- | --- | --- | --- | --- | --- | --- |
| A-B | 108°00'00" | 100.02 | 060°00'00" | 240°00'00" | 50.009 | 86.620 |
| B-C | 108°00'00" | 99.97 | 132°00'00" | 312°00'00" | -66.893 | 74.292 |
| C-D | 108°00'00" | 100.05 | 204°00'00" | 024°00'00" | -91.400 | -40.694 |
| D-E | 108°00'00" | 99.94 | 276°00'00" | 096°00'00" | 10.447 | -99.393 |
| E-A | 108°00'00" | 100.03 | 348°00'00" | 168°00'00" | 97.844 | -20.797 |

## 6.3 Linear Misclosure and Accuracy

ΣDN (eN) = 0.0075 m      ΣDE (eE) = 0.0281 m

Perimeter of traverse = 500.01 m

Linear misclosure = √(eN² + eE²) = 0.0291 m

Linear (relative) accuracy = 1 : 17,183  — well within the acceptable accuracy for a general engineering/topographic control traverse.

## 6.4 Bowditch Corrections and Adjusted DN/DE

| Leg | Corr. DN | Corr. DE | Adj. DN | Adj. DE |
| --- | --- | --- | --- | --- |
| A-B | -0.0015 | -0.0056 | 50.0085 | 86.6142 |
| B-C | -0.0015 | -0.0056 | -66.8945 | 74.2866 |
| C-D | -0.0015 | -0.0056 | -91.4017 | -40.6996 |
| D-E | -0.0015 | -0.0056 | 10.4451 | -99.3981 |
| E-A | -0.0015 | -0.0056 | 97.8426 | -20.8030 |

## 6.5 Final Northings and Eastings

| Station | Northing, N (m) | Easting, E (m) |
| --- | --- | --- |
| A | 1000.000 | 1000.000 |
| B | 1050.009 | 1086.614 |
| C | 983.114 | 1160.901 |
| D | 891.712 | 1120.201 |
| E | 902.157 | 1020.803 |

## 6.6 Area of the Closed Traverse

Using the coordinate (shoelace) method on the final adjusted coordinates: Area = 17,205.46 m² (≈ 1.7205 hectares).

# 7. Conclusion and Recommendation

The group successfully designed, coded and tested a VB.NET programme capable of performing a complete closed-traverse computation, satisfying all the requirements of the assignment: computation of forward/back bearings, latitudes and departures, Bowditch adjustment, final station coordinates, linear accuracy and enclosed area. The sample traverse tested (five stations, equal 108° included angles and near-equal leg lengths) closed both angularly (0° misclosure) and linearly (accuracy of 1:17,183), which is well within acceptable engineering-survey tolerances, confirming that the algorithm and code are functioning correctly.

The programme demonstrates that a modest, well-structured VB.NET Windows Forms application can reliably replace lengthy manual/tabular traverse computations, reducing computational errors and saving considerable time in practice. Presenting the programme as a graphical, interactive tool — rather than a plain console application — also made it far easier to demonstrate the computation live, since field data can be typed directly into the input grid and results appear immediately after clicking "Compute Traverse".

It is recommended that the programme be further extended with stronger input validation (e.g. rejecting angles or distances outside sensible ranges before computation begins), support for both closed-loop and link (open) traverses between two independent control stations, and direct export of the results grid to Excel or PDF for professional survey reporting.

It is further recommended that, where several angle sets are observed, the group extend the code to allow angular misclosure to be automatically distributed among the stations before bearing computation begins, rather than assuming zero misclosure as in the demonstration dataset used in this report.

# References

Uren, J. and Price, W.F. (2010). Surveying for Engineers, 5th Edition. Palgrave Macmillan.

Bannister, A., Raymond, S. and Baker, R. (1998). Surveying, 7th Edition. Pearson Education.

Kavanagh, B.F. (2010). Surveying: Principles and Applications, 9th Edition. Pearson.